### Project Proposal: Smart Home Energy Management System (EMS)

Project Name: SmartHome EMS 2.0

Project Description:
The SmartHome Energy Management System (EMS) 2.0 is an advanced residential energy optimization platform. It leverages IoT devices, edge computing, and AI-driven predictive analytics to minimize energy consumption, improve efficiency, and maintain user comfort in real-time.

Objectives:

- Monitor energy consumption of all major appliances in real-time.

- Predict short-term and long-term energy usage patterns using ML algorithms.

- Provide automated recommendations to reduce peak load and energy costs.

- Enable remote control of lighting, HVAC, and appliances via mobile app.

- Integrate renewable energy sources and battery storage for optimal energy usage.

Scope:

- Residential houses and apartment complexes up to 250 m² per unit.

- Cloud-based analytics with optional local edge processing.

- Mobile app available for iOS and Android platforms.

Technical Architecture:

- Sensors & Devices:

  - Smart plugs with 0.1 W resolution for monitoring power usage.

  - Temperature & humidity sensors (accuracy ±0.5°C, ±3%).

  - Motion and occupancy sensors with <1 s latency.

  - Smart meters for main electrical circuits, supporting up to 32 A.

- Communication:

  - Protocols: MQTT over TLS, Wi-Fi 6, BLE 5.2.

  - Max range: Wi-Fi 6 up to 60 m indoor, BLE up to 20 m.

  - Data rate: 1 Mbps minimum for real-time telemetry.

- Backend Infrastructure:

  - Python FastAPI service for API endpoints.

  - PostgreSQL 15 database with 1 TB initial storage.

  - Redis for caching telemetry data (update every 5 s).

  - Dockerized deployment on Ubuntu 22.04 servers.

- Machine Learning Module:

  - Predictive model using LSTM neural networks.

  - Training dataset: Historical energy usage from 50 homes, 1-year duration.

  - Prediction horizon: up to 24 hours with ±5% accuracy.

  - Retraining frequency: weekly, with automated pipeline using Airflow.

- Frontend:

  - React Native mobile app with offline-first support.

  - Web dashboard for energy analytics and device control.

  - Visualization: real-time graphs, consumption trends, alerts.

Mechanical / Physical Dimensions:

- Smart plug: 55 x 45 x 28 mm

- Sensor hub: 120 x 80 x 40 mm

- IoT gateway: 200 x 150 x 60 mm

- Device operating temperature: -10°C to 50°C

- Maximum supported load: 3680 W per plug

Timeline:

- Phase 1 (Weeks 1–2): Requirement analysis, system design, and component selection.

- Phase 2 (Weeks 3–5): Hardware installation, IoT integration, and calibration.

- Phase 3 (Weeks 6–8): Backend and frontend development, REST API creation.

- Phase 4 (Weeks 9–10): ML model integration, predictive analytics, edge deployment.

- Phase 5 (Week 11–12): User testing, bug fixing, documentation, and deployment.

Budget:

- Hardware: €7,500 (sensors, smart plugs, gateways)

- Development: €20,000 (backend, frontend, ML module)

- Testing & Deployment: €4,000

- Contingency & Misc: €3,000

- Total: €34,500

Deliverables:

- Fully integrated EMS with real-time monitoring and control.

- Predictive energy consumption reports per device and per household.

- Mobile app and web dashboard for control and analytics.

- Complete documentation, maintenance manual, and user training.

Risks & Mitigation:

- Sensor failure: Redundant devices, regular maintenance schedule.

- Data privacy: End-to-end encryption (AES-256), GDPR-compliant cloud storage.

- Network downtime: Local edge caching with automatic sync when online.

- ML inaccuracies: Continuous retraining and anomaly detection.

Conclusion:
The SmartHome EMS 2.0 project provides a complete, technical solution for residential energy optimization. It combines IoT, AI, cloud services, and mobile apps to deliver actionable insights, cost savings, and environmental benefits.